APEX68K.DOC	JAN-11-87

RUNNING APEX ON THE 68000 STRIDE COMPUTER

ASSUMPTIONS

This document assumes that you are already familiar with the Apex/XPL0

system, having used it on another computer such as the Apple II. It also

assumes that you have manuals on Apex, XPL0 and VED.

It is assumed that you have a system similar to the following:

Stride 440 with 1 MB of RAM

Two 5 1/4-inch floppy drives

Wyse WY-30 terminal connected to serial port #0 and in the

factory-set configuration except as follows:

Handshake: DTR (xon/xoff will not work with the editor)

(beware, some terminal cables do not connect pin 20, DTR)

8 data bits, 1 stop bit, no parity

Baud rate: 38400

Right arrow reprogrammed to be a CTRL-U instead of a CTRL-L

(A second terminal may be connected to port #1)

Serial I/O (for comm. with Apple using XGET and XSEND) on port #2

No handshake required (XMODEM protocol is used)

8 data bits, 1 stop bit, no parity

Baud rate: 19200

C.Itoh 8510 or Epson FX-80 printer connected to the parallel port

Version 3.08 of the SDT PROM monitor

Currently, the following Stride hardware is not supported:

No boards other than main mother board and DFM controller board

No Winchester or other hard disk

No serial ports other than ports 0 and 2

No network

No streaming tape (QIC)

No Apex values are saved in the battery-backed-up CMOS memory

BOOTING APEX

Power up the Stride and wait about 5 seconds for the diagnostics to

finish running. You should see something like:

Startup Test [3.08]

Copyright (c) 1982-1985, Stride Micro.

The "PROCESSOR" LED should glow green.

If you are powering on for the first time, you should initialize the

system by typing "IS" otherwise memory parity errors may occur.

If no intelligible message appears on the screen, you may need to perform

the "autobaud" procedure: While holding the space bar down, press the

RESET button on the back of the Stride. You should see: TYPE A '?'.

Typing the question mark will reinitialize the Stride. (For more

information see page 73 in the Stride Owners Manual.)

Insert the APEX SYSTEM DISK into the left floppy drive (unit 0). From the

SDT monitor (after the ">" prompt) type "IF" to initialize from floppy.

The Apex operating system should start. For information on the Apex

commands type "HELP" or see SYNTAX.DOC.

ORIENTATION

Apex provides handlers for "devices" and "units". Devices are byte-

oriented peripherals such as terminals, printers, and modems; units are

block-oriented peripherals such as disk drives and RAM disks. Apex also

provides a file system for units. The current configuration of devices

and units can be displayed by the utility program CONFIG. Here is a

typical configuration:

DEVICE			DESCRIPTION

0		 Terminal handler with buffered keyboard

1		 Terminal handler with unbuffered keyboard

2		 Printer handler

3		 Disk file sequential byte handler

4		 Serial I/O using port #2

5		 Serial I/O using XMODEM protocol

6		 Memory buffer

7		 Null device (bit bucket)

UNIT			DESCRIPTION

0		 Left 5 1/4-inch floppy drive (2560 blocks)

1		 Right 5 1/4-inch floppy drive (2560 blocks)

2		 System RAM disk (2560 blocks)

3		 Work space RAM disk (1500 blocks)

4,5		 Reserved for unique disks

6,7		 Reserved for hard disks

USING THE TEXT EDITOR

For example, to create a new file named "HELLO.TXT" on unit 3, type:

DF 3:HELLO.TXT		(sets the default file name)

MA<space>		(MAkes the default file)

ED<space>		(EDits the default file)

Type in your text. A brief summary of the editing commands will be

displayed if you type:

<esc>?<esc><esc>.

Exit the editor, saving your text, by typing:

<esc>Q<esc><esc>.

To edit your text, restart the editor by typing:

ED<space>

USING THE COMPILER

Create the source code for your XPL program using the editor. The file

name extension should be ".XPL".

To compile and load your program, type the following after the Apex prompt

(APX>):

XPL<space>

CHANGE DEFAULTS (N/Y)? <CR>

LOAD<space>

XPL code is the same as for 6502-based systems with the following

exceptions:

The reserved word 'CHAR' has been dropped; use 'ADDR' instead. Also, in

'CASE' statements 'ELSE' can no longer be used in place of 'OTHER'.

6502 XPL stores low byte, high byte in memory; the 68000 stores high

byte, low byte.

XPL has been expanded to 32-bit integers. This causes the following

incompatibilities:

Integer arrays must RESERVE 4 bytes for every element.

Beware of code which assumes 16-bit integers such as fitting

a 128-integer array into one block on the disk.

HEXOUT displays 8 digits instead of 4.

$FFFF is now equal to 65536, not -1, etc.

See XPL68K.DOC for more details.

USING THE ASSEMBLER

Create the source code for your assembly language program using the

editor. The file name extension should be ".68K".

To assemble and load your program, type the following after the Apex

prompt (APX>):

ASM<space>

CHANGE DEFAULTS (N/Y)? <CR>

LOAD<space>

See ASM.DOC for more details.

FORMATTING NEW DISKS

Diskettes must be "formatted" before they can be used. Apex uses a

16-sector format which is different than the 8-sector format normally

used by Stride.

Currently, there is no format utility which runs under Apex so Stride's

utility must be used. Use the following procedure to format a diskette

with the Apex 16-sector format:

Set the baud rate on the terminal to 9600

Perform the autobaud procedure:

Hold the space bar down and press RETURN on the Stride.

Type a question mark (?)

Insert the floppy disk entitled: "P-System" into the left drive.

From the SDT monitor (after the ">" prompt) type: IF<CR>

From the P-System's main menu type: X

Execute what file? #4:UTIL

Remove the P-System diskette (for safety).

Select (A) On-line Configuration.

Select (G) Low Level Configuration.

Select (D) Right Floppy Details.

Select (C) Sectors per track, and change to 16 (from 8).

Select (D) Bytes per Sector, and change to 256 (from 512).

Select (I) Gap3 parameter, and change to 32 (from 42).

Select (J) Gap3 for format, and change to 50 (from 80).

Select (P) Read after write, and change to "off" (from "on").

press RETURN to exit.

press RETURN to exit back to On-Line Configuration Control.

press RETURN to exit.

Make On-Line changes "Y".

From UTIL Configuration Utility select (C) Floppy Formatter.

Select (R) right drive.

Insert diskette to be formatted, and do it.

Clearly label it as a 16-sector (Apex) diskette.

(It's a good idea to format several diskettes at one time.)

Set the baud rate on the terminal back to 38400

Perform the autobaud procedure

See pages 42 and 107 of the Stride 400 Series Owner's Manual for

additional information.

MAKING BACKUP COPIES

Since it is only a question of time before you accidently destroy the

APEX SYSTEM DISK, it's a very good idea to work with copy of it.

To duplicate an Apex 16-sector diskette, use the DUP utility under Apex

(the Stride utilities won't work):

Boot the APEX SYSTEM DISK.

After the Apex prompt (APX>) type: DUP

Put the source disk in the left drive (unit 0).

Put the copy disk in the right drive (unit 1).

Duplicate from unit 0 to unit 1.

Duplicate 2560 blocks.

#rive (unit 0).

Put the copy disk in the right drive (unit 1).

Duplicate from unit 0 to unit 1.

Dupli