Assembling 3-D shutter glasses

==============================

Richard Ottosen       3/18/94

This is a step by step procedure to make a pair of 3-D shutter glasses from

Alltronics liquid crystal (LC) shutters and saftey goggles. Total parts cost is

about $20.

You will need at least the following tools:

Long nose pliers

Wire cutters

Wire strippers

Soldering iron and solder

Exacto knife

Scissors

1/4" diameter punch

Ruler

All work should be done a clean soft surface. Paper toweling is a good choice.

Try to keep the optical parts free of fingerprints and other contamination.

Whenever working with the LC shutters, make sure that they do not scratch

each other or anything else. The glass is much harder than the polarizing

coating on the shutters, the plastic polarizing strips and the plastic lense of

the goggles.

1.0   Cut the polarizing material to 1" by 2" strips. The polarization must be

at 0 degrees for one peice and 90 degrees for the other.

2.0   Disassemble the safety goggles by first removing the black plastic

"rivet" at the top of the frame. Remove the lense from the frame. Punch a

0.25" diameter hole for the phone jack near the top of the right temple

0.25" from the lense.

3.0   Remove the LC shutter assembly from the plastic by breaking the heat

welds and separating the peices. Pull off the blobs of rubber holding the

wires in place.

4.0   Separate the two shutter plates by cutting and pulling out the U-shaped

layer of foam tape holding them together. Do this by using a sharp knife

being careful not to scratch the LC shutters or knick the wires or

yourself. Hinge the plates apart at the wires and finish removing the

remaining chunks of tape from the glass.

5.0   Carefully peel apart the ribbon cable between the yellow and orange

wires. Make sure that the bond between the wires is what separates and

not the insulation itself. When you are done, the shutters should split

2.0" apart with the brown and red wires in the middle and the orange wire

on the left and the yellow wire on the right with the cable exiting

upwards.

6.0   Dress the cable up and then to the right about 0.25" above the right

shutter. Finish with the shutters spaced 1.0" apart.

7.0   Cut the ribbon cable to be 2.0" long from right edge of the right shutter

(the one with the yellow wire on the right end). Split the wires back

1.0" at the end of the cable.

8.0   Strip all four wires to expose 0.1" of conductor. Pull the insulation

back to expose a total of 0.2" of conductor on each wire. Solder both the

brown and red wires to the sleeve pin of the stereo jack. Solder the

orange wire to the tip pin of the stereo jack. Solder the yellow wire to

the ring pin of the stereo jack.

9.0   Get the polarizing strips and find which goes with which shutter. When

the correct polarizer is selected, there will be a black border around

the edge of shutter plate.

10.0  Check for and remove any fingerprints on the filters, shutters or lense

using an unscented (Kleenex) tissue.

11.0  Take a strip of tape about 3.5" long and apply it to the bottom edge of

the shutter. The tape should exactly cover only the black border created

by the polarizer. Do this with both shutters.

12.0  Get the goggle lense and position it with the nose area down and the "L"

to the left. Horizontaly center the shutters on their respective eye

openings (yellow wire on the right). Align the bottom edge of the tape

with the bottom edge of the lense. Make sure each shutter plate is

perfectly horizontal and properly centered and then press the tape onto

the lense.

13.0  Flatten and straighten the ribbon cable as best as posibble. Position the

jack and ribbon cable for later installation into the goggles frame.

14.0  Slip the polarizing strips under their matching LC shutters. Use a 7"

peice of tape to hold the shutters, polarizing filters and ribbon cable

to the lense. This tape should only cover the black border of the

shutters.

15.0  Use 1.5" lengths of tape to finish holding the shutters in place and to

completely cover the lense. Trim off all excess tape to the shape of the

lense. Rub and press the tape as flat and smooth as possible. Cut a hole

through the tape for the plastic rivet.

16.0  Re-install the lense/shutter assembly into the goggles frame. Bend the

ribbon cable and position the jack to allow this. Bolt the jack into

place. Put the plastic rivet back into the goggles.

17.0  Put a strip of tape over the jack and wires to make them less visable.

18.0  Do a final cleaning of the lense and shutters with tissue.

19.0  Plug the finished glasses into a working LC driver to verify proper

operation.

The glasses are now ready to be used.

-----   Bill of Material   -----

Item Qty Description                                Each     Total

1   1 CABLE, STEREO, PLUG TO PLUG, 3.5MM         3.69      3.69

2   1 CONN, PHONE, JACK, 3.5MM, STEREO, PC       1.00      1.00

3   1 OPTICAL, LCD, SHUTTER, DUAL (SORT OF)     10.00     10.00

4   1 OPTICAL, POLARIZER, PAIR, 1"X2", 90 DEG    1.50      1.50

5   3 SUPPLIES, TISSUE, KLEENEX                  0.01      0.03

6  24 TAPE, CREPE, BLACK, 1" WIDE                0.02      0.48

7   1 TOOL, GOGGLES, SAFETY                      2.00      2.00

---------

$18.70

---------

-----   Source List   -----

Item Qty Description                            Source         Source part num

1   1 CABLE, STEREO, PLUG TO PLUG, 3.5MM     Radio Shack      42-2387

2   1 CONN, PHONE, JACK, 3.5MM, STEREO, PC   Mouser           161-3501

????       1 CONN, PHONE, JACK, 3.5MM, STEREO, PC   Radio Shack	 274-249

3   1 OPTICAL, LCD, SHUTTER, DUAL (SORT OF)  Alltronics	 92L012

4   1 OPTICAL, POLARIZER, PAIR, 1"X2", 90 DE Edmund Scien.    G41,168

5   3 SUPPLIES, TISSUE, KLEENEX              Kimberly-clark   36000 28510

6  24 TAPE, CREPE, BLACK, 1" WIDE            Chartpak         BGCP1-1

7   1 TOOL, GOGGLES, SAFETY                  F. W. Woolworth  TP11076C

The crepe tape comes as a roll 324 inches long. This is enough to do a dozen

pairs of glasses. The roll of tape costs about $5.

The polarizing filters are sold as 20 pieces, each 2" square. To get the two

polarizers with 90 degrees relative polarization requires two squares (and

produces two sets of polarizers). Order the polarizers from:

Edmund Scientific Company		Phone: (609) 573-6250

101 E. Gloucester Pike

Barrington, NJ   08007-1380

The Alltronics shutter is actually two partial shutters. Order one shutter for

each pair of glasses from:

Alltronics				Phone: (408) 943-9773

2300 Zanker Road

San Jose, CA   95131