NOTES ON ASM65.XPL v1.1

Invocation:

>ASM65 filename [/C][/F][/P]

This takes "filename.P65" and produces "filename.BIN"

the switches specify the listing device:

/C   console

/F   file, "filename.LST"

/P   printer.

if no switch is given, no listing is produced. If a listing file is

produced then the binary file is not produced.

The limitations of this version with repect to the Apple ASM65 are these:

1) The program only deals with either the basic 6502 (MODE 0) or the

Mitsubishi processors (MODE $F0). It does not yet deal with the Rockwell Cmos

6502 or with the 65816.

2) There is no CREF.

3) If a listing is requested then a sorted symbol table is produced. The

sort is trivial and slow, but does not take place usually.

\=====================================================================

PROGRAMMER NOTES:

CLASS 1: Operand is address. Can have either zero page or absolute form.

Opcode entry is $xxyy where xx=zpage opcode and yy=absolute opcode. Either

may be missing = 00.

CLASS 2: Operand is address. Address must be $FFxx. Opcode entry is $xx00.

CLASS 3: Operand is branch address. Opcode is $xx00. Distance is TARGET-PC-2.

Fourth characer of mnemonic is bit number to be added into the op.

CLASS 4: Operand is branch address. Opcode is $xx00. Distance is TARGET-PC-2.

CLASS 5: No operand. Opcode is $xx00.

CLASS 6: No operand. Fourth characer of mnemonic is bit number to be added

into the op.

CLASS 7: Two operands, first is zpage address, second is branch address.

Fourth characer of mnemonic is bit number to be added into the op.

In the gen the zpage address is first, then the branch distance which is

TARGET-PC-3

CLASS 8: Operand is one byte constant. Opcode is $xx00.

CLASS 9: Operand is zpage address. Fourth characer of mnemonic is bit number

to be added into the op.

CLASS 10: Two operands. First is one byte constant, second is zpage address.

opcode=$xx00. The gen is constant first, them zpage address.

------------------------------------------------------------------

NOTE: JMP@ AND JSR HAVE NEW SHORT MODES...

WAS "JMP@"  CLASS1  $006C

So, changes to

"JMP@"  CLASS1  $B26C

MITTAB:=[

"JSRS",	2,	$2200		JSRS	FLOC

"JSR@",	1,	$0200		JSR@	ZLOC

"BRA",	4,	$8000		BRA	LABEL

"STP",	5,	$4200		STP

"WAI",	5,	$C200		WAI

"INCA",	5,	$3A00		INCA

"DECA",	5,	$1A00		DECA

"SET",	5,	$3200		SET

"CLT",	5,	$1200		CLT

"COM",	1,	$4400		COM	ZLOC

"TST",	1,	$6400		TST	ZLOC

"LDM#"	10,	$3C00		LDM#	$12,ZLOC

"SWP"	1,	$8200		SWP	ZLOC

"BSA?",	3,	$0300+2i	BSA2	LABEL

"BCA?", 3,	$1300+2i	BCA7	LABEL

"BSM?", 7,	$0700+2i	BSM2	ZLOC,LABEL

"BCM?", 7,	$1700+2i	BCM7	ZLOC,LABEL

"SEA?",	6,	$0B00+2i	SEA2

"CLA?", 6,	$1B00+2i	CLA7

"SEM?", 9,	$0F00+2i	SEM7	ZLOC

"CLM?", 9,	$1F00+2i	CLM2	ZLOC

];