SXOCT.DOC

Here's the "incredible rotating octahedron" demo. If you want to see it,

program the SX on your Parallax demo board with RUN.HEX. Hook an X-Y

scope to A0 and A1 on the demo board, with the X axis on A1. Set 2

volts/div, AC coupling.

The RB0 & RB1 buttons are used to converge the ends of line segments.

This compensates for temperature and component variations, if necessary.

RB2 & RB3 control the tumbling speed.

The demo has three main parts: a D/A virtual peripheral, a technique for

drawing lines, and an example of XPL code.

Lines are drawn by first resolving them into their X and Y components.

The capacitors associated with these components are charged with a

voltage twice of what's necessary to reach the end point of the line.

When enough time has elapsed to charge the caps to half this voltage, the

D/A outputs on the SX are tristated. This moves the beam on the scope to

the end point and leaves it there.

The lines are straight because, although the charging voltage is

nonlinear, the rates at which the X and Y components change are directly

proportional. All line segments take the same amount of time to draw

whether they're short or long. Short segments do look brighter than long

segments, but in the case of a 3-D object this only adds to the realism.

Segments are drawn faster at the beginning than at the end. But the

distortion in brightness is unnoticable since the first half of the

charging curve changes at a fairly constant rate.

Since line segments are drawn using twice the voltage and since the D/A

ranges from 0 to 255 counts, lines are limited to the middle third of

this range, which reduces the resolution that the end points can be

positioned to 85x85.

Another little interesting problem is the lack of an intensity control,

or Z-axis. The octahedron was chosen because it is the only regular

polyhedron that can be drawn without "lifting the pen" or retracing a

line.