SXPL.DOC	JUN-03-98

XPL0 on the SX

XPL0 is a high-level language similar to Pascal. It runs on many

computers besides the PC. This describes the version that runs on Scenix

SX microcontrollers. A detailed description of the XPL0 language is

available in the manual that comes with the PC version, which is

distributed as the file: XPL0-233.ZIP.

XPL0 on the SX is a little different than XPL0 on other processors. It

probably doesn't need to be pointed out, but compared to the PC the SX is

very small. It only has 2K words of ROM. The I2L interpreter uses about

1K, which leaves only about 1K for compiled code. Also the SX has only

136 bytes of RAM of which only 112 are available for the interpreter's

stack and XPL's variables. Arrays must be kept small.

The XPL6 version of the compiler is ideal for the SX because it produces

very compact I2L code--about two-thirds the size of the I2L code on the

PC. However XPL6 is an older version from 6502 days, and it doesn't have

some features that have been added to the PC version in recent years.

Also, to make things fit, floating point operations (reals) and many

intrinsics have been eliminated. You might want to eliminate even more

unused intrinsics to get more space for your compiled code.

The 1K of ROM space available for compiled code is used efficiently. 1K

of 12-bit words equals 1.5K of 8-bit bytes. The new Scenix IREAD

instruction utilizes all 12 bits in a word, not just the 8 bits provided

by the RETLW instruction. Thus the largest compiled program is about 1.5K

bytes. (This space can be increased somewhat by eliminating unused

intrinsics.)

XPL's dynamic memory allocation makes effective use of the limited RAM

space. Only the variables and arrays that are currently needed by the

program (those that are in scope) take up space. When a procedure is

finished, it releases the RAM used by its local variables and makes it

available for the next procedure.

Also to save space, there is only one I/O device, device 0, the serial

port. This is a minor limitation because SXs rarely need generalized

character I/O, such as for a disk drive or printer. In fact the serial

port often is used only for debugging. Instead of character I/O, SXs are

usually used in applications where individual bits need to be read and

written on the I/O ports.

HOW TO RUN AN XPL0 PROGRAM

After you have written a program in XPL, run the batch file called

RUN.BAT. For example, "RUN MYPROG" compiles MYPROG.XPL using XPL6 and

converts the resulting .I2L file into an .ASM file (with lots of DATA

instructions). RUN.BAT appends this .ASM file to I2L.ASM, and the

combination is assembled with Microchip's MPASM. The resulting RUN.HEX

file can be used to program an SX. (RUN.LST can be used to test your

program on SxSim.)

DIFFERENCES BETWEEN SX XPL AND PIC XPL

XPL programs on the SX must be very small (1.5K bytes).

On the SX the I/O ports are not accessible in the memory map (because of

fragmented RAM). The intrinsics Pout (20) and Pin (23) have been added to

compensate for this. Pout also allows access to the Mode register and

TRIS commands, so port bits can be configured as outputs, CMOS levels,

Schmitt triggers, etc.

Currently, there are only three levels of static nesting, instead of the

standard eight. This isn't much of a limitation since it's a rare program

even on the PC that uses more than three levels. (The C language only

allows two levels.)

To save ROM space, I2L error 4 (BADOP) has been removed. There should not

be any floating point (real) operations or 'external' routines because

this error is no longer detected, and the interpreter simply blows up.

The same thing is true for illegal intrinsic (code) numbers: I2L error 5

(BADINT) has also been removed.

An untrapped divide-by-zero error returns -1 instead of 32767. (This is a

minor difference that makes the code a little smaller.)

SX XPL has three intrinsics that PIC XPL doesn't have: CLRWDT=28,

OPTION=29, and SLEEP=30;

DIFFERENCES BETWEEN SX XPL AND PC XPL

XPL programs on the PIC must be small (3-7K).

There is no floating point (no reals).

There is only one I/O device, device 0, serial I/O. There is no circular

buffer device 8.

Calculations cannot be used in 'define's or constant arrays.

There are no declared array dimensions, multidimensional character

arrays, or segment arrays.

There are no external assembly-language subroutines. The same thing is

accomplished by adding your own intrinsics to the I2L.ASM source code.

There is no 'conditional' compile.

The 'addr' operator does not work for subscripted variables.

The up-arrow (^) operator doesn't generate control characters in strings.

A new, compact random number generator is used that probably isn't very

random.

The remainder is always positive. On the PC the remainder has the sign of

the dividend (numerator). On the 6502 the remainder has the sign of the

quotient.

The trap bits are different than the trap bits used by the PC. Here, bit

1 corresponds to I2L error 1, not to I2L error 2 as it does on the PC.

(Trap bits on the PC are inconsistent with all other implementations of

XPL.)

There is no Note intrinsic for compressed ASCII strings. (This was only

available in the 6502 version of XPL6, and not on the PC.)

There are no modules; 'eproc', 'efunc' and 'public', are not used.

Arguments cannot be shown in parenthesis after 'fproc', 'code', etc.

An XPL program should never terminate because there is no operating

system to return to. If a program attempts to return, it simply blows up

(since there is no return to an exit opcode at the beginning of the

compiled code). An 'exit' statement, however, will display any untrapped

errors, or if there are none, it will display the new I2L error: 0. The

program will then restart. Any value returned by 'exit' is ignored.

The Abort intrinsic has been renamed "Reset" since it simply jumps to the

PIC's power-up reset location.

COMMAND LINE SWITCHES

Two command-line switches can be used with XPL6:

/L lists the source code to the screen. This is sometimes helpful in

identifying where a compile error occurs (otherwise, only the preceding

256 characters are displayed, which might not be enough).

/O turns off optimizations. If you suspect that the compiler is not

compiling correctly, you might want to turn off optimizations and bypass

much complexity. There are no known bugs in the optimizing.

There is no warning message if an illegal switch is used.

I2L ERRORS

If an error is detected while an XPL program is running, an I2L error

number is sent to device 0, the serial port. (Unlike in the PC, only the

number is sent; the descriptive text has been eliminated.) These are the

possible I2L errors:

0: 'exit' statement (there's no operating system to exit to)

1: Divide by zero

2: Out of RAM (array too large or too many variables in scope)

3: I/O error (unimplemented device number)

4: Illegal opcode (program blew up)		(REMOVED)

5: Call to an unimplemented intrinsic number	(REMOVED)

15: Store into ROM (can't store into a constant array or string)

POUT & PIN

Because of the scheme used to deal with the fragmented RAM, the I/O ports

are not accessible in the memory map. Thus the intrinsics Pout (20) and

Pin (23) have been provided.

The form of the Pout intrinsic is: POUT(value, port, mode).

When "mode" is 0, the byte in "value" is written to the specified output

"port" location (RA=5, RB=6, RC=7). When "mode" is not 0, it specifies a

control register. For instance, when "mode" is $0F, the data-direction

(tris) register is written.

Here's an example showing how the pullup on bit 7 of RC is enabled:

Pout($7F,7,$E);

It's equivalent to these assembly instructions:

MOVLW	0Eh

MOVWM

MOVLW	7Fh

TRIS	7

The form of the Pin intrinsic is: variable:= PIN(port, mode).

When "mode" is 0, the value on the specified input "port" is returned. If

"mode" is set to $09 and "port" = 6 (RB), the value in the Wake-up

Pending Register (WKPND) is returned. If "mode" is set to $08, the value

in the Comparator Register (CMP) is returned.

Pout and Pin also can write and read RTCC (location 2), as well as any

other location in RAM (but this is not recommended).

HOW IT WORKS

The interpreter (I2L.ASM) resides at the beginning of ROM. The I2L code

generated by the compiler (XPL6) is loaded after the interpreter (at

PROGLO). I2L opcodes are stored using DATA instructions.

The interpreter's stack is implemented using the File Select Register

(FSR) as a stack pointer. The stack grows downward, like on a 6502, and

the stack pointer normally points to an empty location. The low byte is

pushed first (which is technically backwards, but this is the way I2L has

always worked, and it has some small advantages).

The interpreter's variables are in the low end of RAM. The heap resides

above these. The interpreter makes maximum use of the limited RAM by

having the heap and the stack grow toward each other. You can have both a

(relatively) large heap and a large stack, but not at the same time.

The SX has physically separate address spaces for RAM and ROM, and both

start at $0000. The interpreter combines these into a single address

space. Addresses $0000 through $00FF refer to RAM, while all other

addresses refer to ROM. This scheme works because the only ROM addresses

an XPL programmer is concerned about are in the compiled I2L code (such

as a constant array), which is loaded above $0100.

FETCHING I2L INSTRUCTIONS

Unlike in the 14-bit PIC (e.g: 16F84), the RETLW instruction in the SX is

unsuitable for returning I2L instruction bytes because only the locations

in the lower half of each page can be the target address of a CALL

instruction. In other words, only half of ROM could be used. Fortunately

the new IREAD instruction solves this problem, and as a bonus it provides

all 12 bits of a word.

Forgetting the extra four bits for the moment, an I2L instruction is

fetched from ROM at the I2L program counter (I2LPC) as follows:

FETCH	MOVF	I2LPC+1,W	;copy I2LPC into MODE:W

MOVWM			; copy low 4 bits in W into Mode register

MOVF	I2LPC,W		; I2LPC -> W

IREAD			;read instruction at address (MODE:W)

; into MODE:W. Use W and ignore MODE

INCFSZ	I2LPC		;increment I2LPC to point to next instr.

RETP

INCF	I2LPC+1

RETP

The extra four bits can be ignored completely if an XPL program is less

than approximately 1K. If it's greater, the loader goes back to the first

location loaded and starts filling in the high nibbles.

|   I2L.ASM   |

| Interpreter |

|<--12 bits-->|

|    |<8 bits>|

+-------------+

PROGLO	| hi | Start  |

| lo | of I2L |

| hi | code   |

| lo |        |

|    |        |

|    |        |

7FE	|xxxx|I2L code|

7FF  | GOTO  RESET |

+-------------+

UNFRAGMENTING RAM

The SX's fragmented RAM makes it difficult to implement data structures

such as arrays and stacks.

The 136 bytes of RAM are chopped up as shown. There are 8 banks each

containing 16 bytes, and bank 0 has an additional 8 bytes at location 08

(8*16 + 8 = 136). The hatched areas (///) are redundant copies of the

special function registers at 00 and the 8 bytes at 08.

+-------------------------------+

|00 |20/|40/|60/|80/|A0/|C0/|E0/|

|   |///|///|///|///|///|///|///|

|---|///|///|///|///|///|///|///|

|08 |///|///|///|///|///|///|///|

|   |///|///|///|///|///|///|///|

|---+---+---+---+---+---+---+---|

|10 |30 |50 |70 |90 |B0 |D0 |F0 |

|   |   |   |   |   |   |   |   |

|   |   |   |   |   |   |   |   |

|   |   |   |   |   |   |   |   |

|   |   |   |   |   |   |   |   |

+-------------------------------+

A stack pointer can be incremented and decremented, skipping over the

redundant hatched areas, like this:

INCF	FSR		;increment stack pointer

BSF	FSR,4		;make sure it's in the high half of a bank

BCF	FSR,4		;force low half of bank

DECF	FSR		;decrement stack pointer

BSF	FSR,4		;force high half of bank

Values up to and including 16 can be added to FSR like this:

MOVLW	N

ADDWF	FSR

BSF	FSR,4

But adding values greater than 16 (10h) can give incorrect results, for

example:

11h + 10h ! 10h = 31h is correct, but

11h + 1Fh ! 10h = 30h is wrong (50h is correct)

Because of this it's better to calculate addresses ignoring the fragmen-

tation problem, and then convert the resulting "logical" address into a

"physical" address. Here's how this is done:

MOVF	FSR,W		;shift high nibble left

ANDLW	0F0h

ADDWF	FSR

BSF	FSR,4		;force high half of bank

KNOWN BUGS

There is a minor problem with include files that causes some compile

errors to be indicated in the wrong place.

LICENSE

This program is free software; you can redistribute it and/or modify it

under the terms of the GNU General Public License version 2 as published

by the Free Software Foundation.

This program is distributed in the hope that it will be useful, but

WITHOUT ANY WARRANTY; without even the implied warranty of

MERCHANTABILITY or FITNESS FOR A PARTICULAR PURPOSE. See the GNU General

Public License for more details.

You should have received a copy of the GNU General Public License along

with this program (in the file LICENSE.DOC); if not, write to the Free

Software Foundation, Inc., 675 Mass Ave, Cambridge, MA 02139, USA.

Thanks for your interest.

-Loren Blaney

Email: loren_blaney@idcomm.com     Regular Mail: 502 Pine Glade Dr.

Nederland, CO 80466

Updates can be downloaded from:                  USA

http://www.brouhaha.com/~eric/scenix/